- FINECone™ DXF Hints

- Use AutoCAD v12 DXF-format (ASCII).

- DXF MUST be high accuracy: Better than 8 decimal places, best 16

- ONLY normal LINES and ARCS are allowed. No poly-lines etc.

- ALL intersections must meet in ONE point.

- Use Snap in AutoCAD. Example – Line from (x,y)10,21 to start of another arc: line 10,21 (point to start on next arc) END (will indicate a square to lock to the ENDPoint)

- Lines and Arcs to be broken at meeting points (=Intersections)

- Place each component: Cone, Surround, Dust-cap, VC former, VC and spider to be in it’s own layer. The easiest way is to start with the 6_5 Woofer Large Dust Cap.dxf example file and then modify this drawing. In that way the layers are already created.

- No DIMENSIONS, thickness or text at all in DXF drawing.

- Use PURGE in AutoCAD to ensure removal of unwanted items.

- Use Accurate solutions (and NOT FAST) in FINECone (Tools/Program Options/Calculations) to avoid errors due to lack resolution. (components do not follow the whole structure in 3D animation)